- pc.v

`timescale 1ns/1ps

module pc(

input clk,

input reset,

input enable,

input [31:0] pc_next,

output reg [31:0] pc

);

always @(posedge clk or posedge reset) begin

if (reset)

pc <= 0;

else if (enable)

pc <= pc_next;

end

endmodule

- instr_mem.v

`timescale 1ns/1ps

module instr_mem (

input  [31:0] addr,

output [31:0] instr

);

reg [31:0] memory [0:1023];

initial begin

$readmemh("C:/Kittu/M.Tech_NITR_EIE/RISC_V/Codes_new_store/test.hex", memory);

end

assign instr = memory[addr[11:2]]; // word aligned

endmodule

- if_id_reg.v

`timescale 1ns/1ps

module if_id_reg (

input clk,

input reset,

input stall,

input flush,

input  [31:0] pc_in,

input  [31:0] instr_in,

output reg [31:0] pc_out,

output reg [31:0] instr_out

);

always @(posedge clk or posedge reset) begin

if (reset) begin

pc_out    <= 32'b0;

instr_out <= 32'b0;

end

else if (flush) begin

pc_out    <= 32'b0;

instr_out <= 32'b0;   // Insert bubble

end

else if (!stall) begin

pc_out    <= pc_in;

instr_out <= instr_in;

end

end

endmodule

- reg_file

`timescale 1ns/1ps

module reg_file (

input clk,

input we,

input [4:0] rs1,

input [4:0] rs2,

input [4:0] rd,

input [31:0] wd,

output [31:0] rd1,

output [31:0] rd2

);

reg [31:0] regs [0:31];

integer i;

initial begin

for(i = 0; i < 32; i = i + 1)

regs[i] = 32'b0;

end

assign rd1 = (rs1 == 0) ? 32'b0 : ((we && (rd == rs1)) ? wd : regs[rs1]);

assign rd2 = (rs2 == 0) ? 32'b0 : ((we && (rd == rs2)) ? wd : regs[rs2]);

always @(posedge clk) begin

if (we && rd != 0)

regs[rd] <= wd;

end

endmodule

- imm_gen.v

`timescale 1ns/1ps

module imm_gen (

input [31:0] instr,

output reg [31:0] imm

);

wire [6:0] opcode = instr[6:0];

always @(*) begin

case (opcode)

// I-type

7'b0010011,

7'b0000011,

7'b1100111:

imm = {{20{instr[31]}}, instr[31:20]};

// S-type

7'b0100011:

imm = {{20{instr[31]}}, instr[31:25], instr[11:7]};

// B-type

7'b1100011:

imm = {{19{instr[31]}}, instr[31], instr[7],

instr[30:25], instr[11:8], 1'b0};

// U-type

7'b0110111,

7'b0010111:

imm = {instr[31:12], 12'b0};

// J-type

7'b1101111:

imm = {{11{instr[31]}}, instr[31],

instr[19:12], instr[20],

instr[30:21], 1'b0};

default:

imm = 32'b0;

endcase

end

endmodule

- control_unit.v

`timescale 1ns/1ps

module control_unit (

input [6:0] opcode,

input [2:0] funct3,

input [6:0] funct7,

output reg reg_write,

output reg mem_read,

output reg mem_write,

output reg branch,

output reg jump,

output reg alu_src,

output reg mem_to_reg,

output reg [3:0] alu_ctrl

);

always @(*) begin

// defaults

reg_write  = 0;

mem_read   = 0;

mem_write  = 0;

branch     = 0;

jump       = 0;

alu_src    = 0;

mem_to_reg = 0;

alu_ctrl   = 4'b0000;

case (opcode)

// ---------------- R TYPE ----------------

7'b0110011: begin

reg_write = 1;

case ({funct7,funct3})

10'b0000000000: alu_ctrl = 4'b0000; // ADD

10'b0100000000: alu_ctrl = 4'b0001; // SUB

10'b0000000111: alu_ctrl = 4'b0010; // AND

10'b0000000110: alu_ctrl = 4'b0011; // OR

10'b0000000100: alu_ctrl = 4'b0100; // XOR

10'b0000000001: alu_ctrl = 4'b0101; // SLL

10'b0000000101: alu_ctrl = 4'b0110; // SRL

10'b0100000101: alu_ctrl = 4'b0111; // SRA

10'b0000000010: alu_ctrl = 4'b1000; // SLT

10'b0000000011: alu_ctrl = 4'b1001; // SLTU

endcase

end

// ---------------- I TYPE ----------------

7'b0010011: begin

reg_write = 1;

alu_src   = 1;

case (funct3)

3'b000: alu_ctrl = 4'b0000; // ADDI

3'b010: alu_ctrl = 4'b1000; // SLTI

3'b011: alu_ctrl = 4'b1001; // SLTIU

3'b100: alu_ctrl = 4'b0100; // XORI

3'b110: alu_ctrl = 4'b0011; // ORI

3'b111: alu_ctrl = 4'b0010; // ANDI

3'b001: alu_ctrl = 4'b0101; // SLLI

3'b101: alu_ctrl = funct7[5] ? 4'b0111 : 4'b0110; // SRAI/SRLI

endcase

end

// ---------------- LOAD ----------------

7'b0000011: begin

reg_write  = 1;

mem_read   = 1;

alu_src    = 1;

mem_to_reg = 1;

alu_ctrl   = 4'b0000;

end

// ---------------- STORE ----------------

7'b0100011: begin

mem_write = 1;

alu_src   = 1;

alu_ctrl  = 4'b0000;

end

// ---------------- BRANCH ----------------

7'b1100011: begin

branch   = 1;

alu_ctrl = 4'b0001;

end

// ---------------- JAL ----------------

7'b1101111: begin

reg_write = 1;

jump      = 1;

end

// ---------------- JALR ----------------

7'b1100111: begin

reg_write = 1;

jump      = 1;

alu_src   = 1;

end

// ---------------- LUI ----------------

7'b0110111: begin

reg_write = 1;

end

// ---------------- AUIPC ----------------

7'b0010111: begin

reg_write = 1;

end

endcase

end

endmodule

- hazard_detection_unit.v

module hazard_detection_unit(

input id_ex_mem_read,

input [4:0] id_ex_rd,

input [4:0] if_id_rs1,

input [4:0] if_id_rs2,

output reg pc_stall,

output reg if_id_stall,

output reg id_ex_flush

);

always @(*) begin

pc_stall    = 0;

if_id_stall = 0;

id_ex_flush = 0;

if (id_ex_mem_read &&

(id_ex_rd != 0) &&

((id_ex_rd == if_id_rs1) ||

(id_ex_rd == if_id_rs2))) begin

pc_stall    = 1;

if_id_stall = 1;

id_ex_flush = 1;

end

end

endmodule

- id_ex_reg.v

`timescale 1ns/1ps

module id_ex_reg (

input clk,

input reset,

input flush,

// DATA

input  [31:0] pc_in,

input  [31:0] rd1_in,

input  [31:0] rd2_in,

input  [31:0] imm_in,

input  [4:0]  rs1_in,

input  [4:0]  rs2_in,

input  [4:0]  rd_in,

input  [2:0]  funct3_in,

// CONTROL

input  reg_write_in,

input  mem_read_in,

input  mem_write_in,

input  mem_to_reg_in,

input  alu_src_in,

input  branch_in,

input  jump_in,

input  [3:0] alu_ctrl_in,

// OUTPUTS

output reg [31:0] pc_out,

output reg [31:0] rd1_out,

output reg [31:0] rd2_out,

output reg [31:0] imm_out,

output reg [4:0]  rs1_out,

output reg [4:0]  rs2_out,

output reg [4:0]  rd_out,

output reg [2:0]  funct3_out,

output reg reg_write_out,

output reg mem_read_out,

output reg mem_write_out,

output reg mem_to_reg_out,

output reg alu_src_out,

output reg branch_out,

output reg jump_out,

output reg [3:0] alu_ctrl_out

);

always @(posedge clk or posedge reset) begin

if (reset || flush) begin

pc_out <= 0;

rd1_out <= 0;

rd2_out <= 0;

imm_out <= 0;

rs1_out <= 0;

rs2_out <= 0;

rd_out  <= 0;

funct3_out <= 0;

reg_write_out <= 0;

mem_read_out  <= 0;

mem_write_out <= 0;

mem_to_reg_out <= 0;

alu_src_out <= 0;

branch_out <= 0;

jump_out <= 0;

alu_ctrl_out <= 0;

end

else begin

pc_out <= pc_in;

rd1_out <= rd1_in;

rd2_out <= rd2_in;

imm_out <= imm_in;

rs1_out <= rs1_in;

rs2_out <= rs2_in;

rd_out  <= rd_in;

funct3_out <= funct3_in;

reg_write_out <= reg_write_in;

mem_read_out  <= mem_read_in;

mem_write_out <= mem_write_in;

mem_to_reg_out <= mem_to_reg_in;

alu_src_out <= alu_src_in;

branch_out <= branch_in;

jump_out <= jump_in;

alu_ctrl_out <= alu_ctrl_in;

end

end

endmodule

- forwarding_unit.v

`timescale 1ns/1ps

module forwarding_unit(

input  [4:0] id_ex_rs1,

input  [4:0] id_ex_rs2,

input  [4:0] ex_mem_rd,

input        ex_mem_reg_write,

input  [4:0] mem_wb_rd,

input        mem_wb_reg_write,

output reg [1:0] forwardA,

output reg [1:0] forwardB

);

always @(*) begin

forwardA = 2'b00;

forwardB = 2'b00;

//////////////////////////

// EX hazard (priority)

//////////////////////////

if (ex_mem_reg_write &&

(ex_mem_rd != 0) &&

(ex_mem_rd == id_ex_rs1))

forwardA = 2'b10;

if (ex_mem_reg_write &&

(ex_mem_rd != 0) &&

(ex_mem_rd == id_ex_rs2))

forwardB = 2'b10;

//////////////////////////

// MEM hazard

//////////////////////////

if (mem_wb_reg_write &&

(mem_wb_rd != 0) &&

!(ex_mem_reg_write &&

(ex_mem_rd != 0) &&

(ex_mem_rd == id_ex_rs1)) &&

(mem_wb_rd == id_ex_rs1))

forwardA = 2'b01;

if (mem_wb_reg_write &&

(mem_wb_rd != 0) &&

!(ex_mem_reg_write &&

(ex_mem_rd != 0) &&

(ex_mem_rd == id_ex_rs2)) &&

(mem_wb_rd == id_ex_rs2))

forwardB = 2'b01;

end

endmodule

- alu.v

`timescale 1ns/1ps

module alu (

input [31:0] a,

input [31:0] b,

input [3:0] alu_ctrl,

output reg [31:0] result

);

always @(*) begin

case (alu_ctrl)

4'b0000: result = a + b;                       // ADD

4'b0001: result = a - b;                       // SUB

4'b0010: result = a & b;                       // AND

4'b0011: result = a | b;                       // OR

4'b0100: result = a ^ b;                       // XOR

4'b0101: result = a << b[4:0];                 // SLL

4'b0110: result = a >> b[4:0];                 // SRL

4'b0111: result = $signed(a) >>> b[4:0];       // SRA

4'b1000: result = ($signed(a) < $signed(b));   // SLT

4'b1001: result = (a < b);                     // SLTU

default: result = 32'b0;

endcase

end

endmodule

- branch_unit.v

`timescale 1ns/1ps

module branch_unit (

input [31:0] a,

input [31:0] b,

input [2:0] funct3,

output reg take_branch

);

always @(*) begin

take_branch = 1'b0;

case (funct3)

3'b000: take_branch = (a == b);                       // BEQ

3'b001: take_branch = (a != b);                       // BNE

3'b100: take_branch = ($signed(a) < $signed(b));      // BLT

3'b101: take_branch = ($signed(a) >= $signed(b));     // BGE

3'b110: take_branch = (a < b);                        // BLTU

3'b111: take_branch = (a >= b);                       // BGEU

default: take_branch = 1'b0;

endcase

end

endmodule

- ex_mem_reg.v

`timescale 1ns/1ps

module ex_mem_reg (

input clk,

input reset,

input flush,

input  [31:0] alu_result_in,

input  [31:0] write_data_in,

input  [31:0] branch_target_in,

input         zero_in,

input  [4:0]  rd_in,

input  [2:0]  funct3_in,

input  reg_write_in,

input  mem_read_in,

input  mem_write_in,

input  mem_to_reg_in,

input  branch_in,

input  jump_in,

output reg [31:0] alu_result_out,

output reg [31:0] write_data_out,

output reg [31:0] branch_target_out,

output reg        zero_out,

output reg [4:0]  rd_out,

output reg [2:0]  funct3_out,

output reg reg_write_out,

output reg mem_read_out,

output reg mem_write_out,

output reg mem_to_reg_out,

output reg branch_out,

output reg jump_out

);

always @(posedge clk or posedge reset) begin

if (reset || flush) begin

alu_result_out <= 0;

write_data_out <= 0;

branch_target_out <= 0;

zero_out <= 0;

rd_out <= 0;

funct3_out <= 0;

reg_write_out <= 0;

mem_read_out <= 0;

mem_write_out <= 0;

mem_to_reg_out <= 0;

branch_out <= 0;

jump_out <= 0;

end

else begin

alu_result_out <= alu_result_in;

write_data_out <= write_data_in;

branch_target_out <= branch_target_in;

zero_out <= zero_in;

rd_out <= rd_in;

funct3_out <= funct3_in;

reg_write_out <= reg_write_in;

mem_read_out <= mem_read_in;

mem_write_out <= mem_write_in;

mem_to_reg_out <= mem_to_reg_in;

branch_out <= branch_in;

jump_out <= jump_in;

end

end

endmodule

- data_mem.v

`timescale 1ns/1ps

module data_mem (

input clk,

input mem_read,

input mem_write,

input [2:0] funct3,

input [31:0] addr,

input [31:0] write_data,

output reg [31:0] read_data

);

reg [7:0] memory [0:4095];

always @(posedge clk) begin

if (mem_write) begin

case (funct3)

3'b000: memory[addr] <= write_data[7:0]; // SB

3'b001: begin                             // SH

memory[addr]   <= write_data[7:0];

memory[addr+1] <= write_data[15:8];

end

3'b010: begin                             // SW

memory[addr]   <= write_data[7:0];

memory[addr+1] <= write_data[15:8];

memory[addr+2] <= write_data[23:16];

memory[addr+3] <= write_data[31:24];

end

endcase

end

end

always @(*) begin

if (mem_read) begin

case (funct3)

3'b000: read_data = {{24{memory[addr][7]}}, memory[addr]}; // LB

3'b001: read_data = {{16{memory[addr+1][7]}},

memory[addr+1], memory[addr]};       // LH

3'b010: read_data = {memory[addr+3], memory[addr+2],

memory[addr+1], memory[addr]};        // LW

3'b100: read_data = {24'b0, memory[addr]};                 // LBU

3'b101: read_data = {16'b0, memory[addr+1], memory[addr]}; // LHU

default: read_data = 32'b0;

endcase

end

else read_data = 32'b0;

end

endmodule

- mem_wb_reg.v

`timescale 1ns/1ps

module mem_wb_reg (

input clk,

input reset,

input [31:0] mem_data_in,

input [31:0] alu_result_in,

input [4:0]  rd_in,

input reg_write_in,

input mem_to_reg_in,

output reg [31:0] mem_data_out,

output reg [31:0] alu_result_out,

output reg [4:0]  rd_out,

output reg reg_write_out,

output reg mem_to_reg_out

);

always @(posedge clk or posedge reset) begin

if (reset) begin

mem_data_out <= 0;

alu_result_out <= 0;

rd_out <= 0;

reg_write_out <= 0;

mem_to_reg_out <= 0;

end

else begin

mem_data_out <= mem_data_in;

alu_result_out <= alu_result_in;

rd_out <= rd_in;

reg_write_out <= reg_write_in;

mem_to_reg_out <= mem_to_reg_in;

end

end

endmodule

- rv32i_pipeline_top.v

`timescale 1ns/1ps

module rv32i_pipeline_top(

input clk,

input reset

);

//////////////////// GLOBAL CONTROL ////////////////////

wire pc_stall;

wire if_id_stall;

wire id_ex_flush;

wire if_flush;

//////////////////// IF STAGE ////////////////////

wire [31:0] pc;

wire [31:0] pc_next;

wire [31:0] instr;

//////////////////// IF/ID ////////////////////

wire [31:0] if_id_pc;

wire [31:0] if_id_instr;

//////////////////// ID STAGE ////////////////////

wire [4:0] rs1;

wire [4:0] rs2;

wire [4:0] rd;

wire [31:0] rd1;

wire [31:0] rd2;

wire [31:0] imm;

wire reg_write;

wire mem_read;

wire mem_write;

wire mem_to_reg;

wire alu_src;

wire branch;

wire jump;

wire [3:0] alu_ctrl;

//////////////////// ID/EX ////////////////////

wire [31:0] id_ex_pc;

wire [31:0] id_ex_rd1;

wire [31:0] id_ex_rd2;

wire [31:0] id_ex_imm;

wire [4:0] id_ex_rs1;

wire [4:0] id_ex_rs2;

wire [4:0] id_ex_rd;

wire [2:0] id_ex_funct3;

wire id_ex_reg_write;

wire id_ex_mem_read;

wire id_ex_mem_write;

wire id_ex_mem_to_reg;

wire id_ex_alu_src;

wire id_ex_branch;

wire id_ex_jump;

wire [3:0] id_ex_alu_ctrl;

//////////////////// EX ////////////////////

wire [1:0] forwardA;

wire [1:0] forwardB;

wire [31:0] alu_in1;

wire [31:0] alu_in2_raw;

wire [31:0] alu_in2;

wire [31:0] alu_result;

wire branch_taken;

wire [31:0] branch_target;

//////////////////// EX/MEM ////////////////////

wire branch_taken_final;

wire [31:0] ex_mem_alu_result;

wire [31:0] ex_mem_write_data;

wire [31:0] ex_mem_branch_target;

wire ex_mem_branch_taken;

wire [4:0] ex_mem_rd;

wire [2:0] ex_mem_funct3;

wire ex_mem_reg_write;

wire ex_mem_mem_read;

wire ex_mem_mem_write;

wire ex_mem_mem_to_reg;

wire ex_mem_branch;

wire ex_mem_jump;

//////////////////// MEM ////////////////////

wire [31:0] mem_data;

//////////////////// MEM/WB ////////////////////

wire [31:0] mem_wb_mem_data;

wire [31:0] mem_wb_alu_result;

wire [4:0] mem_wb_rd;

wire mem_wb_reg_write;

wire mem_wb_mem_to_reg;

//////////////////// WB ////////////////////

wire [31:0] wb_data;

////////////////////////////////////////////////////////////

//////////////////////// IF STAGE //////////////////////////

////////////////////////////////////////////////////////////

pc PC(

.clk(clk),

.reset(reset),

.enable(~pc_stall),

.pc_next(pc_next),

.pc(pc)

);

instr_mem IM(

.addr(pc),

.instr(instr)

);

////////////////////////////////////////////////////////////

/////////////////////// IF/ID REG //////////////////////////

////////////////////////////////////////////////////////////

if_id_reg IF_ID(

.clk(clk),

.reset(reset),

.stall(if_id_stall),

.flush(if_flush),

.pc_in(pc),

.instr_in(instr),

.pc_out(if_id_pc),

.instr_out(if_id_instr)

);

////////////////////////////////////////////////////////////

//////////////////////// ID STAGE //////////////////////////

////////////////////////////////////////////////////////////

assign rs1 = if_id_instr[19:15];

assign rs2 = if_id_instr[24:20];

assign rd  = if_id_instr[11:7];

reg_file RF(

.clk(clk),

.we(mem_wb_reg_write),

.rs1(rs1),

.rs2(rs2),

.rd(mem_wb_rd),

.wd(wb_data),

.rd1(rd1),

.rd2(rd2)

);

imm_gen IG(

.instr(if_id_instr),

.imm(imm)

);

control_unit CU(

.opcode(if_id_instr[6:0]),

.funct3(if_id_instr[14:12]),

.funct7(if_id_instr[31:25]),

.reg_write(reg_write),

.mem_read(mem_read),

.mem_write(mem_write),

.branch(branch),

.jump(jump),

.alu_src(alu_src),

.mem_to_reg(mem_to_reg),

.alu_ctrl(alu_ctrl)

);

////////////////////////////////////////////////////////////

//////////////////// HAZARD DETECTION //////////////////////

////////////////////////////////////////////////////////////

hazard_detection_unit HDU(

.id_ex_mem_read(id_ex_mem_read),

.id_ex_rd(id_ex_rd),

.if_id_rs1(rs1),

.if_id_rs2(rs2),

.pc_stall(pc_stall),

.if_id_stall(if_id_stall),

.id_ex_flush(id_ex_flush)

);

////////////////////////////////////////////////////////////

//////////////////////// ID/EX REG /////////////////////////

////////////////////////////////////////////////////////////

id_ex_reg ID_EX(

.clk(clk),

.reset(reset),

.flush(id_ex_flush| if_flush),

.pc_in(if_id_pc),

.rd1_in(rd1),

.rd2_in(rd2),

.imm_in(imm),

.rs1_in(rs1),

.rs2_in(rs2),

.rd_in(rd),

.funct3_in(if_id_instr[14:12]),

.reg_write_in(reg_write),

.mem_read_in(mem_read),

.mem_write_in(mem_write),

.mem_to_reg_in(mem_to_reg),

.alu_src_in(alu_src),

.branch_in(branch),

.jump_in(jump),

.alu_ctrl_in(alu_ctrl),

.pc_out(id_ex_pc),

.rd1_out(id_ex_rd1),

.rd2_out(id_ex_rd2),

.imm_out(id_ex_imm),

.rs1_out(id_ex_rs1),

.rs2_out(id_ex_rs2),

.rd_out(id_ex_rd),

.funct3_out(id_ex_funct3),

.reg_write_out(id_ex_reg_write),

.mem_read_out(id_ex_mem_read),

.mem_write_out(id_ex_mem_write),

.mem_to_reg_out(id_ex_mem_to_reg),

.alu_src_out(id_ex_alu_src),

.branch_out(id_ex_branch),

.jump_out(id_ex_jump),

.alu_ctrl_out(id_ex_alu_ctrl)

);

////////////////////////////////////////////////////////////

//////////////////// FORWARDING UNIT ///////////////////////

////////////////////////////////////////////////////////////

forwarding_unit FU(

.id_ex_rs1(id_ex_rs1),

.id_ex_rs2(id_ex_rs2),

.ex_mem_rd(ex_mem_rd),

.ex_mem_reg_write(ex_mem_reg_write),

.mem_wb_rd(mem_wb_rd),

.mem_wb_reg_write(mem_wb_reg_write),

.forwardA(forwardA),

.forwardB(forwardB)

);

////////////////////////////////////////////////////////////

//////////////////////// EX STAGE //////////////////////////

////////////////////////////////////////////////////////////

assign alu_in1 =

(forwardA==2'b10) ? ex_mem_alu_result :

(forwardA==2'b01) ? wb_data :

id_ex_rd1;

assign alu_in2_raw =

(forwardB==2'b10) ? ex_mem_alu_result :

(forwardB==2'b01) ? wb_data :

id_ex_rd2;

assign alu_in2 = id_ex_alu_src ? id_ex_imm : alu_in2_raw;

alu ALU(

.a(alu_in1),

.b(alu_in2),

.alu_ctrl(id_ex_alu_ctrl),

.result(alu_result)

);

branch_unit BU(

.a(alu_in1),

.b(alu_in2_raw),

.funct3(id_ex_funct3),

.take_branch(branch_taken)

);

// Determine if we are doing a JALR (alu_src=1) or JAL (alu_src=0)

// JALR uses the ALU result (rs1 + imm), but forces the LSB to 0.

assign branch_target = (id_ex_jump && id_ex_alu_src) ? {alu_result[31:1], 1'b0} : (id_ex_pc + id_ex_imm);

// Calculate the return address (PC + 4) to save in 'rd'

wire [31:0] return_addr = id_ex_pc + 4;

// If it's a jump, send the return address down the pipeline instead of the math result

wire [31:0] final_ex_val = id_ex_jump ? return_addr : alu_result;

////////////////////////////////////////////////////////////

//////////////////////// EX/MEM REG ////////////////////////

////////////////////////////////////////////////////////////

ex_mem_reg EX_MEM(

.clk(clk),

.reset(reset),

.flush(1'b0),

.alu_result_in(final_ex_val),

.write_data_in(alu_in2_raw),

.branch_target_in(branch_target),

.zero_in(branch_taken),

.rd_in(id_ex_rd),

.funct3_in(id_ex_funct3),

.reg_write_in(id_ex_reg_write),

.mem_read_in(id_ex_mem_read),

.mem_write_in(id_ex_mem_write),

.mem_to_reg_in(id_ex_mem_to_reg),

.branch_in(id_ex_branch),

.jump_in(id_ex_jump),

.alu_result_out(ex_mem_alu_result),

.write_data_out(ex_mem_write_data),

.branch_target_out(ex_mem_branch_target),

.zero_out(ex_mem_branch_taken),

.rd_out(ex_mem_rd),

.funct3_out(ex_mem_funct3),

.reg_write_out(ex_mem_reg_write),

.mem_read_out(ex_mem_mem_read),

.mem_write_out(ex_mem_mem_write),

.mem_to_reg_out(ex_mem_mem_to_reg),

.branch_out(ex_mem_branch),

.jump_out(ex_mem_jump)

);

////////////////////////////////////////////////////////////

//////////////////////// MEM ///////////////////////////////

////////////////////////////////////////////////////////////

data_mem DM(

.clk(clk),

.mem_read(ex_mem_mem_read),

.mem_write(ex_mem_mem_write),

.funct3(ex_mem_funct3),

.addr(ex_mem_alu_result),

.write_data(ex_mem_write_data),

.read_data(mem_data)

);

////////////////////////////////////////////////////////////

//////////////////////// MEM/WB ////////////////////////////

////////////////////////////////////////////////////////////

mem_wb_reg MEM_WB(

.clk(clk),

.reset(reset),

.mem_data_in(mem_data),

.alu_result_in(ex_mem_alu_result),

.rd_in(ex_mem_rd),

.reg_write_in(ex_mem_reg_write),

.mem_to_reg_in(ex_mem_mem_to_reg),

.mem_data_out(mem_wb_mem_data),

.alu_result_out(mem_wb_alu_result),

.rd_out(mem_wb_rd),

.reg_write_out(mem_wb_reg_write),

.mem_to_reg_out(mem_wb_mem_to_reg)

);

////////////////////////////////////////////////////////////

//////////////////////// WRITEBACK /////////////////////////

////////////////////////////////////////////////////////////

assign wb_data =

mem_wb_mem_to_reg ?

mem_wb_mem_data :

mem_wb_alu_result;

////////////////////////////////////////////////////////////

//////////////////////// PC UPDATE /////////////////////////

////////////////////////////////////////////////////////////

// PC UPDATE LOGIC

// Trigger a branch if a conditional branch is taken, OR if it's an unconditional jump

assign branch_taken_final = (id_ex_branch && branch_taken) || id_ex_jump;

assign pc_next = branch_taken_final ? branch_target : pc + 4;

assign if_flush = branch_taken_final;

endmodule

test.hex

00A00093    addi x1,  x0, 10

01400113    addi x2,  x0, 20

002081B3    add  x3,  x1, x2

00118233    add  x4,  x3, x1

00202023    sw   x2, 0(x0)

00002283    lw   x5, 0(x0)

00528333    add  x6,  x5, x5

00700393    addi x7,  x0, 7

00108663    beq  x1,  x1, 12

06400413    addi x8,  x0, 100

0C800493    addi x9,  x0, 200

04D00513    addi x10, x0, 77

001505B3    add  x11, x10, x1

40258633    sub  x12, x11, x2

001666B3    add  x13, x12, x1

Waveforms:

- RAW Hazard

- Load use hazard

- Control Hazard